B.G. SAPOZHNIKOV

R e s i s t i v i t y   m e t h o d   w i t h o u t   g r o u n d i n g

Resistivity measurements - the classical  method of electric prospecting - to the present day provide one of the most efficient geophysical techniques for exploration of the media differing in resistivity to the depths  ranging from the first tens of centimeters to the first hundreds of meters.

The resistivity method is based on excitation and measurements of d.c. or low-frequency (to first tens of hertz) electric fields in the media under investigation with the use  of  transmitting ("AB") and receiving  ("MN") lines grounded at the ends. The method is used both for electric sounding and profiling. It has a well-developed interpretation theory with extensive software and does not involve heavy expenses for equipment and field work.

Typical applications of the resistivity method are geological prospecting, engineering   investigations, environmental and archaeological studies.

One of the limitations of the resistivity method resides in its impracticability in areas with the surface cover unfavorable for grounding current and measuring electrodes (screes, frozen and loose ground, snow and ice cover, dry sand, asphalt and concrete pavements).

The alternative electric methods for the mentioned  depths  of investigations are ground penetrating radar (GPR) and electromagnetic (EM) methods (Heimmer, 1992).  In contrast to the resistivity measurements the majority of these methods do not employ grounded electrodes and consequently can be applied with any conductivity of the surface cover.

The GRP  method  is based on the high frequency ( > 10-50 MHz) electromagnetic reflections from dielectric boundaries. The typical depth of investigation is not more than several meters. The method, which only recently has become the practice in electromagnetic prospecting, is used successfully for exploration of horizontally stratified media in shallow engineering and archaeological investigations. The disadvantages of this method are the limited depth of investigation, the ambiguity of the results in laterally inhomogeneous media, high cost of the equipment, software and service.

The EM  methods are based on excitation of electromagnetic fields in the media under study and measuring primarily magnetic components of  the field (sometimes in combination with electric components).

In low-frequency domain (at frequencies of up to first  kHz) these methods are used to directly explore high-conductivity local targets, such as metallic ores and graphite deposits, and estimate their conductivity. Targets of medium and low conductivity cannot be detected. In high-frequency domain (up to first MHz) the sensitivity range of the EM methods to resistivity variations grows, yet the depth of investigation decreases significantly due to the skin effect. A grave disadvantage of these methods both in the high-frequency and particularly in the low-frequency domain, is low sensitivity to the poorly conducting targets such as quartz veins, house foundations, etc. Should  the need arise to differentiate the rock resistivities in a wide range of values, the EM methods  give way to  the  resistivity method (e.g. Zhamaletdinov et al., 1995). The substitution of the resistivity method by EM methods can be justified only when resistivity measurements are impracticable due to the surface conditions adverse to grounding.

The proposed technique is aimed at increasing the efficiency of the resistivity method by extending its application into the areas with unfavorable conditions of grounding;  cutting down labour expenditures by reducing the size of field teams; widening the scope of functional potentialities of the method, in particular by virtue of vector measurements of the electric field in the air (Sapozhnikov, 1986, 1994, 1996; Ermokhin, Sapozhnikov, 1996; Glazunov, Sapozhnikov, 1996; Nakhabtsev et al., 1985, 1990, 1992;  Bolshakov et al., 1996;  Dukarevich et al., 1996).

When the electric field is measured in the air near the earth-air interface,  the obvious physical basis for the proposed improvement of the resistivity method is the known equality of the tangential components of the electric field on both sides of the mentioned interface. This allows to substitute contact measurements of the electric field using galvanically grounded measuring electrodes  by  contactless  measurements with the use of receiving electric antennae (electrodes) which either have no galvanic contact with the ground  at  all or the contact is extremely poor.

Depending on the spatial resolution required and the type of array, antennae of different designs can be used, e.g. a telescopic symmetrical air antenna with an effective length of up to 1.5 m, or an asymmetrical antenna made up of  insulated wires of up to 100 m in length creeping over the ground (a “snake”-like antenna). In some types of arrays the operator caring a microvoltmeter may serve himself as one of measuring electrodes with the other electrode being grounded, etc.

When the exciting field frequency  is sufficiently  high (500-2000 Hz)  use can be made  of galvanically nongrounded transmitting lines in the form of unclosed insulated wires creeping over the surface ("capacitive" line) or closed loops ("inductive" line).

The basic equipment essential for realization of the proposed improvements comprises  microvoltmeters or input amplifiers ("active electrodes") with very high input impedance (input capacitance: 0.1 to 30 pF; input resistance: 0.1 to 20 GOhm), and transmitters producing output voltage stepped up to 1-1.5 kV.

The following drawings serve to illustrate the theoretical feasibility of measuring the electric field in air by the resistivity method.

Shown in Fig.1 is the pattern of the d.c. field caused by two grounded current electrodes "A" and "B" located on the horizontal earth-air interface. The equipotential lines are shown by black lines and lines of force of the electric field intensity (the current density lines) by red lines. Letters "M" and "N" denote grounded electrodes of the potential "MN" line. This figure is commonly used to explain the theoretical basis of the resistivity method. The accompanying formulae describe the electric field in the ground. The field  pattern, however, is not complete for it does not show electric field lines in the air. The air may be considered as  an ideal insulator which prevents the galvanic current flow; therefore, the current density lines cannot describe the electric field in  the air.  Does this bear evidence of nonexistence of  the electric field in the air ?

In order to give the answer to this question  let  us  consider Fig.2. The electric field pattern and the accompanying equations presented here describe the most general case of field calculations where a horizontal interface divides two homogeneous conductive media with resistivities "r1" and "r2". The equations have been derived through the use of the method of the images and the boundary conditions of the Maxwell equations:  the equalities of the normal components of the current densities and the tangential components of the electric field intensities. Hence, calculating the limit of the expression (2) for r1 ® ¥ and taking r2 to be equal to the ground  resistivity it is easy to derive the equations (1) which now describe electric fields in the air and in the ground.  The pattern of the static electric field in the air (equipotential lines of field intensity and lines of force of the  field  intensity) turned out to be a mirror image of the pattern of the stationary electric field in the ground, as shown in Fig.2. As can be seen from Fig.2, the horizontal component of the electric field at the central part of the "AB" line decreases  equally slow as the observation point moves away in both directions from the earth-air interface. It means that the height dependence of this field component is extremely small and can be neglected when observations are made at heights comprising 1-5 % of length of the “AB” line. The vertical component of the electric field in the air at the central part of the "AB" line is much smaller than the horizontal component.

Fig.3 and Fig.4 illustrate the solution of another problem: measurements of the electric field in the air.

Fig.1 shows a conventional grounded "MN" line, and Fig.3 shows the same line on an enlarged scale. The equivalent electrical circuit of the receiving  line (Fig.3)  includes  e.m.f. "DU" (of the potential difference of the measuring electrodes) and the microvoltmeter input resistance "RIN" and the ground  resistances of the measuring electrodes "RM" and "RN". From the equation in Fig.3 it is inferred that the input voltage "Uin",  measured by the microvoltmeter, is practically equal to the potential difference "DU" at favorable  grounding conditions when the  sum  of  resistances (RM + RN) is sufficiently small compared to the microvoltmeter input resistance. From the measured  value "DU" and known  electrode separation "a" the intensity "Ex" of the electric field horizontal component can be easily calculated: Ex = DU /a.

The proposed nongrounded receiving line made up of two similar line electrodes "M" and "N" is shown in Fig.2, and in Fig.4  the same line is shown on an enlarged scale. The electrode separation distance "a" is here also the distance between the electrode centres (“a” -  is the effective length of an antenna). The line made up of two lengths of insulated wire may be either laid out directly on the ground surface  (a "snake" line) or raised  to an considerable height (an air antenna). The receiving line electrodes acquire the potentials of the equipotential lines through the centres of the electrodes. The equivalent electrical circuit of the line (Fig.4) includes the e.m.f. "DU" and the capacitance voltage divider formed by the microvoltmeter input impedance and self-capacitances "CM" and "CN" of the measuring electrodes.

When selecting  the measuring array parameters, consideration must be given to the presence of  interferences  which hinder measurements. When employing the grounded receiving line, which has been discussed, the following interferences occur:

- natural and artificial d.c. and a.c. fields;

- electromotive force of electrode polarization of electrochemical character.

Unfortunately, a well-known technique of interference rejection based on interference signal compensation in the absence of the transmitting line current, does not provide a means for sufficiently accurate account of interference instability. For this reason, direct current measurement arrays have very limited sensitivity. as a consequence, starting in 1950-60 the resistivity prospecting was carried out using low-frequency equipment operating in the frequency range from the first unities to first tens of hertz. The sensitivities of measuring arrays therewith increased more than  an order of magnitude, because the necessity for interference compensation vanished, and narrow-band filtering enabled effective rejection of interference due to artificial and natural fields of unwanted frequencies.

Let us consider the interferences which hinder the electric field measurements in the air. The e.m.f. of electrode polarization is absent here,  while the natural, essentially vertical, static field reaches 100-150 V/m in proximity to the ground surface and is very unstable in magnitude. This makes impossible resistivity  measurements in the air using tolerable direct current strengths. Therefore, it is also expedient to employ the nongrounded  array for a.c. resistivity measurements.

In order to select an operating frequency let us  consider  the equivalent circuit of the array in Fig.4.  At d.c. the desired signal "Uin" is zero. As the frequency rises the circuit transforms into a frequency-independent voltage divider. At a sufficiently small input capacitance "Cin" the desired  signal is practically equal to the e.m.f. value "DU" being measured. Calculations of the electric field intensity can be made  from the equation which was given before for the grounded receiving line. When selecting the optimal operating frequency, conflicting consideration must be given to the following discrepancies: on the one  hand, lower frequencies provide a greater depths of investigation of the resistivity method, on the other hand, higher frequencies enable a more effective rejection of vibrational interferences which are specific for the array in question. These interferences are due to the e.m.f. of electrization of the ungrounded line insulators. The interferences are considerable in magnitude and have an essentially low-frequency spectrum. The investigations revealed that optimal frequencies range from 20 to 2500 Hz. A basic frequency selected for developed system "ERA" is 625 Hz.

Let us focus on the nature of anomalous electric fields in the air to be measured by the resistivity method. Fig.5 shows a vertical high-resistivity layer a limited depth which is a local causative body of anomaly. In an normal electric field with horizontal polarization the layer is polarized, i.e. its vertical sides acquire stationary electric charges. This is the cause of anomalous electric currents and corresponding electric fields. Since galvanic currents must not penetrate into the insulator (air), static electric charges appear on the surface preventing the current flow from the ground into the air. The distribution of these charges is fully determined by the current-flow pattern in the ground, and hence, the electric field of these charges in the air reflects this pattern. Fig.5 shows the curves of the horizontal "Ex" and vertical "Ez" components of the anomalous electric field in the air  in the vicinity of the earth-air interface. It  may be noted that qualitatively  these curves are similar to the curves of the "Hx" and "Hz" components of an  anomalous magnetic field over a vertical layer magnetized in the Earth's magnetic field in proximity to the equator.

To summarize, it has been shown that normal and anomalous electric  fields in the air  may be subjects of investigations  by the resistivity method. It has been also shown that a nongrounded  array  for a.c. measurements is feasible.  Of course,  the pattern of the electric field in the air, which makes allowance  for  the application of alternating current, differs from that being discussed. Calculation of array electromagnetic fields,  performed under a more rigorous statement of the problem, show that the differences are not of qualitative character.

The proposed advancement of the resistivity method,  known as "the procedure of electric  field contactless measurements", has been successfully used in Russia for more than 20 years in  geological prospecting, engineering investigations, environmental and archaeological studies. Below are given the illustrations and case history of the field work  carried out with the use of new procedure and equipment.

Fig.6-8 show high identity of the field results obtained with the new and conventional resistivity measurement techniques. With the aim of comparison of the results the measurements were made in the areas where grounded transmitting and receiving lines could be applied.

In Fig.6 (Yakutia, 1966) is shown the definition of a quartz vein in granites and gneisses with the use of a gradient array. The current electrode separation is 500 m; both grounded and nongrounded receiving lines have an effective length of 5 m. The conventional and new techniques employ operating frequencies of 22.5 and 625 Hz respectively. The current line of the nongrounded array is a half-loop made up of “snake”- like insulated wire spread on the ground in the form of a square of side of 500 m. Application of the new procedure allowed to reduce the size of the field team to one person.

Fig.7 shows an example of mapping of fault zones in metamorphic and sedimentary rocks using a dipole-dipole array with a set separation of 50 m (Sayani, 1975). The receiving and transmitting lines of the ungrounded array are line electrodes made of a “snake” insulated wire. The operating frequencies  of the two alternative techniques are the same as in Fig.6. With the technique the size of the field team is reduced to 2 persons.

Fig.8 (The Amur river region, 1996) presents some results of vertical electric sounding (VES) carried out in winter on the Zeya river ice with the aim of engineering investigations of the designed dam site of the Tchagoyanski hydroelectric station. The vertical electric sounding array is a three-electrode system with an “MN” line length of 1 m. The “AO” separations were varied from 2 to 240 m (15 separations). Distance “OB” from the sounding center “O” to the remote electrode “B” is 1200 m; the average ice thickness is 1.4 m. Measurements performed at hole No. 4 with the conventional procedure at operating frequencies of 4.88 and 625 Hz (fig.8a) demonstrated the frequency dependence is practically nonexistent. Grounding of the current and potential lines was done by dipping the electrodes into the water through the holes drilled in ice . Measurements by the new technique were carried out at a frequency of 625 Hz with the movable electrode “A” ungrounded. The comparison of the two alternative techniques (fig.8a) disclosed practical identity of the measurement results beyond the range of influence of the ice layer. The enabled the whole VES program  (over 40 soundings) to be accomplished by the new technique with significant reduction in labor consumption required for drilling numerous holes to ground the movable electrode. Vertical electric sounding measurements by the new technique can be carried by one operator.

Fig.9 (The Crimea, 1992) shows the results of experimental work performed at an archaeological site with the use of a telescopic air antenna in an effort to investigate the height dependence of gradient array observations. As illustrated, the  average signal level practically does not alter with the height of measurements growing from zero (grounded potential line 1 m in length) to 75 cm (air antenna with an effective length of 1 m). The local features of the observation plots, however, are smoothed notably therewith. The same relationship exist also for antennas with shorter effective lengths.

Fig.10-11 (Transbaikal region, Yakutiya, 1976-77) present the results of experimental work with the use of the telescopic antenna in vector measurements of the electric field in the air. These experiments proved for the first time that the vertical anomalous component of the electric field, which was never measured by the resistivity method before, can be measured on the ground surface (Fig.10). The possibility of vector measurements of the rotary electric field was also  shown (Fig.11).

Fig. 12 (Kareliya, 1980) gives an example of the field work performed with the use of the telescopic antenna both in winter and summer time. Resistivity prospecting with a gradient array was carried out as part of a geophysical mapping package, which included magnetic prospecting (Fig. 13) and measurements of the natural d.c. field (Fig.14). The purpose of the work was to map Proterozoic metamorphic rocks at a site of the lake Onega shore. Winter-time electric and magnetic survey was carried out on the ice. The geological map of the site constructed from the survey results (Fig.15) disclosed its folded structure.

The results of similar surveying accomplished on a much larger area (15 sq.km) are shown in Fig.16-18 (Kareliya, 1978). The survey performed in winter-time over snow and ice allowed to cover the entire survey area without gaps, such as lakes or impassable  swamps, inevitable in summer. Electric prospecting was carried out with a gradient array using a “snake” ungrounded “MN” line with an effective length of 20 m. The main structures of the geological map (Fig.18) constructed from the interpreted results of the electric (Fig.16) and magnetic (Fig.17) survey were confirmed by drilling.

REFERENCES

Heimmer, D.H., 1992: Near-surface, high resolution geophysical methods for cultural resource management and archaeological investigation. Geo-Recovery Systems, Inc., 400 Corporate Circle, Suite B Golden, Colorado. 143 pp.

Zhamaletdinov, A.A.,  Ronning,  J.S.,  Vinogradov, Yu.A. 1995: Electrical  profiling by the MISC and Slingram methods in the Pechenga-Pasvik  area.  Nor. geol. unders.  Special  Publ. 7. 333-338.

Sapozhnikov, B.G., 1986:  Electrical profiling with nongrounded transmitting and receiving lines.  The Applicant's Thesis for the Degree of the Candidate of Science (Tech.),  Moscow. 295 pp. (In Russian).

Sapozhnikov, B.G.,  1994:  Electrical antenna - a new tool for application of  the  resistivity  method  in archaeology.  Abstract of paper,  presented at the International conference  of  the application of natural sciences methods in archaeology (The European Network PACT,  Russian Academy of Science,  State Hermitage, Saint-Petersburg State University) in S.-Petersburg,  27-30 November, 1994. Program and abstracts. 200 pp. 172.

Sapozhnikov, B.G., 1996: Resistivity method without grounding. Report, presented at the 21st General Assembly of the EGS in The Hague,  6-10 May, 1996. Annales Geophysical, Supplement of Volume 14, 1996.

Ermokhin, K.M.,   Sapozhnikov,  B.G.,  1996:  3D-modeling  of electric fields for the resistivity method in  air.  Abstract of paper, presented at the 21st General Assembly of the EGS in The Hague,  6-10 May,  1996. Annales Geophysical, Supplement of Volume 14, 1996.

Glazunov, V.V., Sapozhnikov, B.G., 1996: Vertical Gradient of electric field measurement on base of air resistivity method. Abstract of paper, presented at the 21st General Assembly of the EGS in The Hague,

6-10 May, 1996. Annales Geophysical, Supplement of Volume 14, 1996.

Nakhabtsev,  A.S.,  Sapozhnikov,  B.G., Yablouchansky,  A.I.  1985: Electric profiling with  non-grounded transmitting and receiving lines. Leningrad. 96 pp. (In Russian).

Orthogonal electrical sounding with non-grounded working lines. Leningrad, 1990. The editor: Nakhabtsev, A.S. 115 pp. (In Russian).

Nakhabtsev, A.S. 1992: Low-frequency electrical methods with linear sources of garmonic field.  S.-Petersburg. 359 pp. (In Russian).

Bolshakov, D.K., Modin, I.N., Sapozhnikov, B.G., Shevnin, V.A. 1996:  Non-contact resistivity measurements. Report, presented at 58-th Annual EAGE conference, 3-7 June  1996, Amsterdam, The Netherlands. P-51, 2 pp.

Dukarevich, L.I., Rybin, V.C., Sapozhnikov, B.G., 1996: "ERA"- active electrode EM/resistivity system. Abstract of paper, presented at the 21st General Assembly of the EGS in The Hague,  6-10 May, 1996. Annales Geophysical, Supplement of Volume 14, 1996.